[Number System] 2’s Comp. conversion

Positive Numbers

- Comp = original (补码=原码)

- Original = comp (原码=补码)

Negative Numbers

- Comp = reverse(abs(original)) + 1 (补码=正数的反码+1)

- Original = -reverse(comp-1) (原码=补码-1/反转/再加负号)

Negative Numbers

1. 统一加减法：减法可以转换为加法来处理。

2. 简化溢出处理：正负溢出方式相同，简化溢出的检测逻辑。

3. 避免两个零：假若不用补码，则00002, 10002 都代表0

Overflow detection

- Positive + Positive = Negative

- Negative + Negative = Positive

Multiplication

- Left-shift multiplicand B

- Right shift multiplier Q (so each time we can take Q[0])

- A (product) = A + B * Q[0]

Floating Point Notation

- Significand: Normalized to start with 1 (±1.bbbb…b×2E)

- Bias = 2^(k-1)-1,  k is the number of bits of the “biased exponent”

- Bias Exponent:  Biased exponent=E+bias

Decimal Fraction -> Binary Float Point:

−29.21875 → 32-bit IEEE floating-point

Therefore -29.21875=-11101.0011=-1.11010011×24

MIPS

J-type instructions

- Only 26bits address

- [leftmost 4 bits of PC] + [26bit address] + [00, 2 zeros]

I-type instructions

- The immediate value “imm” only have 16 bits

- Zero extend leftwards to extend it to 32 bits

R-type instructions

- $rs = $rt + $rd. 5 Bit each register (since we have 32 in total)

- Shamt used in shift function

MIPS Instruction Cautions & Notes

Bit-wise operations

- ORI: Must be non-negative (2’s comp. meaningless after zero extend). ORI with zero can load non-negative constant to register

- ANDI: sign irrelevant (AND with zero padding always be 0). Good for clearing registers.

- XORI: sign irrelevant. Good for flipping specific bits of a register (by setting some bits of the immediate value to 1)

- NOR: equivalent to NOT (anything NOR 0, flips its bit)

- SLL: implement No-Op (sll $0, $0, 0)

- SRL: must be non-negative (due to zero padding on the left)

- SRA: sign extend left (extend with leftmost bit)

Arithmetic operations

- ADDU: NOT unsigned. It means overflow is ignored

- ADD: overflow is detected

- ADDIU: overflow ignore, sign extended

- Same for SUBU / SUB (overflow ignored / detected)

- MULTU: multiply unsigned (no overflow check)

- MULT: multiply signed (no overflow check)

- MFHI D: move ‘hi’ register -> ‘D’ register

- MFLO D: move ‘lo’ register -> ‘D’ register

- DIVU: division for unsigned. Remainder -> hi ; Quotient -> lo

Memory Access Operations

- LW $1, imm($2):       $1 <- RAM [ $2 + SignExtend(imm) ]

- SW $1, imm($2):       RAM[ $2 + SignExtend(imm) ] <- $1

- LUI $1, imm:              copy imm to upper 16bits of $1

- ORI $1, $0, imm:       copy imm to lower 16bits of $1

- .data Directive: Declare variables & constants. Data section start at 0x10000000.

- .word Directive: Allocates a 4Bytes. (.data 69 / .data 1, 2, 3, 4)

Control Flow Operations

- J / JAL, PC + 4 before jump, thus need Branch Delay SLot

Subroutine

- $ra (return address) <- PC + 8

- $sp (stack pointer): points to stack top (smallest address)

- Stack push: subu $sp, $sp, 4 -> sw $t0, ($sp) --- ($sp) equiv 0($sp)

- Stack pop: lw $t0, ($sp) -> addu $sp, $sp, 4

Boolean Algebra

Number of Possible Switching Functions: 22N (N input variables)

Boolean Function Duality (Right Side of Above Table)

- Replace AND with OR ; Replace OR with AND

- Replace constant 1 with 0 ; Replace 0 with 1

- New function has exact opposite output.

Combinational Logic

Multiplexer (MUX)  reduce wire usage    Demultiplexer (DeMUX)

Truth Table

Karnaugh Map (K-Map)

- Bit order: 00 01 11 10 (both row and column)

- Group biggest blocks of 1s.

- “X” or “don’t care” don’t necessarily need to be circled. Only circle them when it simplifies things

Minterm: Product term in SOP form mi,mj,…

Maxterm: Sum term in POS form Mi,Mj,…

Quine-McKluskey tables

- Idea: if 2 minterms differ only in one position, can be simplified

- AB + AB’ = A(B + B’) = A

Procedure

- Group minterms according to the number of 1’s

- Check adjacent groups for simplification (group 1 & 2,  2 & 3 …)

- Merge minterms only differ by one position. Denote the differing position with ‘-‘ (canceled)

- Repeated terms can be cancelled

- Repeat 1~5, Until Can’t merge. Now we get prime implicant: a minterm that cannot be further simplified with other minterms

Prime Implicant Table: each prime implicant covers some minterms

上面表格中如有minterm仅被1个prime implicant 覆盖，那么这个prime implicant是essential prime implicant

Sequential Circuit

- Not only dependent on current (comb. circuits) but also past behavior (i.e., storage element / memory)

- Output (next state) = current input + current state.

- Flip-flops: bistable device (two stable states – doesn’t change without external input). Outputs: Q,Q complements each other.

SR Flipflops

Edge Triggered (or Clocked) S-R Flip-Flop

D Flip-Flop

JK Flip-Flop

- Unlike SR Flip-Flop (which can’t take S=1, R=1 as input)

- JK Flip-Flop allows all combination of input (including 1 1)

- Output tick between 1, 0 each clock cycle when J=1 K=1

8-bit Parallel Registers

Counter (register whose value incr. by 1 every tick)

- For counter with N flip flops, the value ranges from 0 ~ 2N-1

- Asynchronous counter: state of the FF will NOT change same time

- Synchronous counter: state of the FF changes at the same time

Ripple Counter

Finite State Machine

- Registers = Memory Component

- Comb. logic = State transition Function

FSM Model -> Circuit

- Model the problem using FSM

- Construct excitation table of the circuit

Columns:

- Q: current state

- X: Input

- Qnext: next state

- Input to Flip-Flop

Basically:

- FSM -> State table

- FF -> FF ExTable

- Circuit table = FF ExTable + State Table

- K-map simplification: All pairs of (State Q) vs (Flip-Flop Input)

Cache Mapping

Cache Performance

- Miss Rate = 1 – Hit Rate   ( % of mem accesses not found in cache)

- Hit Time = Time to deliver word from cache -> memory

- Miss Penalty = extra time needed (to access memory) due to miss

Memory Address

- Block Offset: index of each word within the block.

- Index: indicates which cacheline

- Tag: unique identifier of blocks mapped to same cacheline (blocks of same cacheline have different tags)

Calculations:

- N bit address -> 2n addressable words

- K words per block -> M=2n/K blocks

- Offset: log2K bits --- log (Block Size) bits

- Index: log2C bits --- log (Num. of Cacheline) bits

- Tag: n-log2K-log2C --- Total Length of Address – Length of Offset – Length of Index

Memory Access Logic

- Row Access: 11 lines + RAS -> select 2048 rows

- Col Access: 11 lines + CAS -> select 2048 columns

- Read/write: controlled by WE / OE pins.

Error Detection and Correction

- Codeword = original data + parity bits

- D_min = minimum hamming distance among codewords

- Max Detectable Error: D_min – 1 bits

- Max Correctable Error: Floor ( (D_min – 1 ) / 2) bits

Contiguous Memory Allocation

Fixed Partition

- Memory divided into one or more fixed-sized pattern

Variable Parition

- Division not fixed size (process allocated just enough memory), thus no internal fragmentation

Fragmentation

- Internal: unused space within partition (fixed partition only)

- External: enough total space, but non-contiguous

- Solution: compaction (defragmentation)

Non-Contiguous Memory Allocation

Paging

- Page Table: translate page -> Frames

- Page: logical memory divided into fixed-size blocks (process view)

- Frame: physical mem divided into fixed-size blocks (actual)

- Virtual Address = { Page Number, Page Offset }

- Page Offset has log2(page & frame size)

Segmentation

- Program divided into variable-szied segments

- Virtual Address = { Segment Number, Segment Offset }

- Segment Table:

- Base Address : Physical starting address of the segment

- Segment Limit: Length of that segment

- Physical Address = Base Address + Segment Offset

Magnetic Disk

- Constant Angular Velocity (CAV) – LEFT

- Multiple Zoned Recording -- RIGHT

- Access Must Specify: Cylinder, Head, Sector, Transfer Size, Memory Address

- Access Delay = Seek Time + Rotational Delay

- Format of a Track [ Sector ] [ Sector ] [Sector ] …

- Format of Sector [gap] [ID field] [gap] [Data Field] [gap]

- ID Field: [Synth Byte] [Track] [Head] [Sector] [CRC]

Solid-State Drive (SSD)

- Erase entire block before write

- Write data in new space then garbage collect ‘deleted’ space

I/O Modules - Function

- Control and Timing: Proc request; I/OM check & return device status; transfer data if device ready

- Processor Communication: Command (control bus, CPU -> I/OM), Data (data bus), Status (BUSY, READY, error conditions)

- Device Communication: I/O -> devices. Unique address for each device (depending on Memory Mapped I/O or Isolated I/O)

- Data Buffering: I/OM buffers data to handle different transfer rates.

- Error Detection: soft error (parity bit) OR hard error (paper jam)

I/O Modules – Technique

Programmed I/O (PIO): I/O Device -> CPU -> RAM

- CPU -> IOM sends command

- IOM perform action, sets status register when finished

- Pooling: CPU periodically checks status register to check completion

Interrupt-Driven I/O: I/O Device -> CPU -> RAM

- Same as PIO. Except after command send, CPU continue other work

- IOM perform action, interrupt CPU when ready

- CPU save state (register, PC, etc), process interrupt, restore state.

Direct Memory Access (DMA): I/O device -> DMA -> RAM

- CPU tells DMA what to do (e.g., device addr, memory loc, words)

- DMA transfer entire block of data. Interrupt CPU when finished.

- Cycle Stealing: DMA steals some cycles from CPU. CPU slowed.

- Single bus, detached DMA. Use bus twice

- CPU suspend twice

- Single bus, integrated DMA. Use bus once

- CPU suspend once

- Separate I/O Bus.

- CPU suspended once

- Use Bus once